PREMMIA MANAGEMENT SUPPORT TOOLS

README.DOC Rel. 1.00

Five programs are provided which demonstrate and use the management

support features of the AST Premmia.  These utilities are designed to be

run from an MS-DOS command prompt (the infamous "C>").  They seem to

work under the Windows 3.1 "DOS Box" but success is not guaranteed.

The first four programs are designed to provide the system administrator

with information about the machines connected to a network.  The fifth

program permits the system BIOS to be updated without having to use

a floppy diskette.  The following gives a detailed description of each

of these programs.

The administrator information programs use two features of the Premmia:

the serial number and the asset tag.  The serial number is set at

the factory and is the same one that appears on the back of the

system enclosure.  The asset tag is left blank. It is intended for

use by administrators to do equipment tracking, etc.  Two of the

programs, AT and SNAT, provide the ability to read and modify these

values.

With AT, you can view the asset tag and serial number and change the

asset tag.  This utility is intended for use by system administrators.

With SNAT, you can change both the asset tag and the serial number.  The

serial number is stored in battery backed-up CMOS RAM and may be lost if

the system's battery fails.  Otherwise, the serial number should not be

modified.  This utility is intended for use by technical support.

Another two other programs form a part of what we call the Very Simple Node

Monitor.  They work together to record and then view some statistics,

including the asset tag, serial number, processor type, and cache status

of systems.  If used across a network, a database can be built of the

assets contained in various Premmia systems.

Conventions:

Example text appears in double quotes (" to ").  The quotes are not part

of the example.  Multi-line examples are separated from descriptions by

lines of "-*-*-*-*...".  Again, these lines are not part of the example.

-----------------------------------------------------------------

AT - Asset Tag Utility

-----------------------------------------------------------------

The asset tag is 16 characters long and is intended for use by

administrators to mark systems according to whatever requirements they

may have.  The AT program provides access to the asset tag.  It is also

visible in Setup.

AT can be used in 3 ways.

AT /?

gives information on how to use the program.

AT

prints the system's serial number and asset tag.  You can save the

results to a file by using "AT >file.txt".

AT newtag

makes the system's asset tag "newtag".  The new tag can contain anything

other than space, "<", ">", or "|".

Example:

AT GeraldFord

would set the asset tag to "GeraldFord".  The program will also tell you

what the old tag was (if there was one).

-----------------------------------------------------------------

SNAT - Serial Number and Asset Tag Utility

-----------------------------------------------------------------

The serial number is a factory-set value.  It is useful when requesting

technical support and is a unique way of identifying a system.  As noted

above, the serial number configured at the factory is the "system" or

enclosure serial number.  If the motherboard is moved from one enclosure

to another, the serial number should be changed.  Also, if the battery

fails, the serial number will become blank.  SNAT provides technicians a

way to change the serial number and asset tag.  The serial number may be

viewed in Setup or using the AT program, above.

SNAT can be used in the following ways:

SNAT

gives a brief summary of how to use the program

SNAT A newvalue

changes the Asset Tag (hence the "A") to newvalue.

SNAT S newnumber

changes the Serial Number (hence the "S") to newnumber.

SNAT A newvalue S newnumber

changes both the Asset Tag and Serial Number at one time.

newvalue and newnumber don't have to be just numbers.  You can use any

characters except ">", "<", "|", and space.  The "A" and the "S" can be

upper or lower case.

Example:

SNAT a Fred's s 012346

sets the Asset Tag to "Fred's" and the serial number to "012346".

-----------------------------------------------------------------

Very Simple Node Monitor (VSNM)

-----------------------------------------------------------------

VSNM consists of two parts: a recorder and a viewer.  The recorder is

installed in the log-in scripts of systems ("nodes" in network-speak) to

be monitored.  The viewer turns the recorded information into a report.

The systems are tracked by serial number.

-----------------------------------------------------------------

VSNMREC - Very Simple Node Monitor Recorder

-----------------------------------------------------------------

VSNMREC records information about a Premmia system in a database.  The

programs setup looks more complicated than it is.  Its syntax is:

VSNMREC <file> <network_address> <user_id> <full_name>

where:

<file> is the name of the database.  The user must be able to write to

the file.  In a network environment, this will almost always be a shared

network file.

<network_address> is the physical address of the node on the network.

<user_id> is the user's login id.  On most systems, this is the sequence

of characters the user must type, along with his password, to gain

access to the net.

<full_name> is the user's full name.  This can be several names (or

parts of a name), separated by spaces, or contain other useful

information.

For Novell NetWare, use, e. g.

#VSNMREC q:database.vsn %P_STATION %LOGIN_NAME %FULL_NAME

in the login scripts.  NetWare will fill in the correct values.

VSNMVIEW and VSNMREC aren't very picky about the contents of the last 3

(or more) parameters.  As long as they don't contain >, <, or |, it's

OK.

For other networks, the information provided by NetWare may not be

available.  It can be installed "manually" instead:

VSNMREC q:database.vsn 1234 CalC Calvin Coolidge

The Node Id would be "1234".  The User Id is "CalC" and the full name is

"Calvin Coolidge".

-----------------------------------------------------------------

VSNMVIEW - Very Simple Node Monitor Viewer

-----------------------------------------------------------------

Syntax:

VSNMVIEW <file>

where <file> is the same file name that appears in VSNMREC lines.

The following is an example of a report generated by VSNMVIEW.

-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

AST Very Simple Node Monitor (VSNM) Viewer  Rel. 1.00.

Copyright (C) AST Research, Inc., 1993.  All Rights Reserved.

VSNMVIEW Report for file "b:database.vsn".

Report generated Wed Jan 13 17:08:26 1993

Serial number: PrFv0026, Asset Tag:  ABCInc000846.

System:  AST Premmia.

Processor:  486DX2 at 33 MHz (*2 internally).

Memory:  8MB RAM, 256KB cache.

Revs:  BIOS: 1.00.  Flash Loader: 1.00.

Node:  415253001234.

User name:  Dan Fortmuller.  (DanF)

Last modified:  Fri Jan 08 16:58:25 1993

Serial number: PrFv0859, Asset Tag:  ABCInc000844.

System:  AST Premmia.

Processor:  486DX at 33 MHz.

Memory:  4MB RAM, 256KB cache.

Revs:  BIOS: 1.00.  Flash Loader: 1.00.

Node:  415253009876.

User name:  Larry Sheppard.  (LarryS)

Last modified:  Mon Jan 11 9:08:55 1993

- - - END OF REPORT - - -

-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

The example report describes two systems.  The system descriptions are

not printed in any particular order.  Most of the lines are

self-explanatory.

The "Serial number" is the system serial number as set at the factory or

by SNAT (above).  The "Asset Tag" can be modified using AT or SNAT

above.

The processor speed (33 MHZ) is printed before being "doubled" in an e.

g. 486DX2. If the speed is doubled, "(*2 internally)" is appended to the

end of the processor line.

The "Memory:" line only concerns memory on the motherboard (i. e.

provided by the integrated SIMMs).  The cache size does not include any

cache internal to the processor itself.

The text in parenthesis on the "User name" line (e.  g.  "(DanF)") is

the user id.  The rest of the text on that line ("Dan Fortmuller") is

the full name.

The "Last modified" line is the last time any of this information

changed which is not necessarily the last time the user logged in.

To print the results to a printer use, e. g.

VSNMVIEW database.vsn >LPT1

To save it as a file so it may be incorporated into a report (created,

say, in a word processor):

VSNMVIEW database.vsn >file.txt

Import the file into your word processor as "ASCII" or "Text".

The fifth program permits the system BIOS to be updated without using

a floppy diskette.  Instead of using a floppy diskette this program

reads a file that contains the new system BIOS and automatically updates

the BIOS, beeps a success or failure code and halts the machine.  By

using this program the system administrator can place a new BIOS file

on the network and each user can update the BIOS without having to

handle floppy diskettes.

DEVICE=UPBIOS.EXE [flags] filename [filename ...]

This program is a device driver that will update the BIOS and/or

MODULE support code.  The program should be executed first in the

CONFIG.SYS file.  This driver will not take any memory space.  The

filenames passed to the driver must reside on the users local hard

disk.  By copying the BIOS image from a network drive to the local

hard disk the BIOS updates placed on the network will propagate

to the hard disk and then the user will update his BIOS on the

next reboot.

If an update is necessary performed the user will be prompted for

a password (either the administrator or user password may be used)

then the system will display instructions about what is going on.

In about 20 seconds the system will beep twice, indicate the update

has been completed and the user may reset the system using the

reset button.

The following flags may be used to alter the operation.

/?        Display a help message indicate what the command line syntax is.

/q[uiet]  Display only error messages and prompts.

/f[orce]  Force an update even if the update is an older revision.  But

do not do an update if the update is the same as what is running.

/n[eed]   Display the status of the update, but do not update.  This

is used as a diagnostic tool to determine why an update

is or is not taking place.

/h[alt]   Halt if the user does not enter the proper password within

five tries.  This can be used by the system administrator

to force the user to update the BIOS before using the system.

If the system beeps more than twice after an update the problem can

be determined by looking up the beep code in the users manual.

The file format for the update is not the "FLASH.AST" that is usually

found on update diskettes.  Instead the file is a diskette image

that is used by the AST BBS.

Example:

In CONFIG.SYS

Device=C:\AST\UPBIOS.EXE /halt C:\AST\BIOS.NEW

(the reset of the config.sys file follows)

In AUTOEXEC.BAT or SYSTEM LOGIN SCRIPT

COPY N:\PREMMIA\BIOS.NEW C:\AST\BIOS.NEW

Disclaimer:

Where used (Windows, NetWare, Intel processor names), companies'

trademarks are their own.  We try to be compatible with all of them.

End of README.DOC